Rough River 2010 notes

- Rough River Lodge’s # is 270-257-2312 Donna is Jeff Roberts contact

- Jeff Roberts “Gold Rush” was down with cracked undercarriage mounting frame. Office 615-355-7575, Cell 615-406-8651

- John Wigney flies with set of spark plugs, spare tube and jacking block

- Creighton Smith and Jim Woolnough located in Brattleborough VT 802-863-6067 building a Classic with a 912 with dual FI and two PC680’s and aft crank driven alternator. Jim used to hand lap lenses, has plenty of auto and boat fabrication experience.

- Erich likes 25PSI on main tire, with 18 it rubbed caliper. His caliper is located closer to 6 o-clock He had problems with cooling and elongated intake slots that worked for him. He fabricated cowl. He has 1+1/4” CF thin wall CF legs with inline roller skate wheels. He bent one CF tube but it never broke. He said he posted a few times how he painted his plane. He rolled on K-36 and in hard to reach areas brushed it on. He painted his wings vertical. He said if you get a run, you can take it off while still wet with tape, then re spray. He uses dawn (a few drops0 to wet sand with and clean things up. Color sanding he does with small blocks begin with something like 400 and goes to 1500. He is using a Murphy Rebel molding to mount his oil tank. I noticed his left aileron had a lot of slop in it (1/4” plus).

- John Lawton has with one aileron neutral the other one is down by about 1”. He did that on purpose to achieve better performance, but said it will spin like a top. He said not for the novice. Bud told me that the Europa wing replicates an elliptical wing and the tips should not be doing any lifting. He thought dropped ailerons (washin) would not help things.

- Kim Prout has flat wingtips not rounded, his Dad thought it would be better than rounded.

- Kevin is using ¾” fiberglass reinforced gear legs; he just recently snapped one on a hard landing.

- Jim Brown replaces his gear legs every 2 to 3 years or when they take a set with stock purchased from ACS. He had cooling problems and enlarged the radiator intake scoop, which helped, but he lost 2 knots.

- Bill Lindsey has flown 11 test flights on Europas. He said put a dot on windscreen and when you look at end of runway, put centerline between dots!

- Fred has a third 914 pump mounted on aux. Tank and pumps fuel into cobra.

- Kim said that the intercooler works on older style TCUs as it prevents intervention. He said Eric tucker said that the 914 turbo impellor must be balanced to 130K, not 100K like many turbo shops can do it to.

- Bud is a Airmaster dealer and I think a service provider. He said they are working on internal to prop shaft brushes for the 332 that will not only last longer but will allow you to add a belt driven alternator. He is also working a blended ferrule fairing that is mounted to the spinner that replicated what they are doing on turbo props. He mentioned 5% better whatever that means. I imagine that could perhaps help out with on the ground cooling? He has no time frame when this project may take place. He flew in a 2 bladed Airmaster with Sensenich blades on his 914 trigear. It cruises at 100 knots. I don’t know what power setting he was using at take off but plane didn’t accelerate as good as other birds.

- GLIDER WINGS. The glider wings failed at the root spar. Resolution is to increase height of the spar (web). I think the procedure went something like having a mold and filling it with CF roving. Ampreg 21 must be used which is a problem as getting it into this country at the moment. I think there is also 8 CF BID put on the web, but I could have taken poor notes and there is 8 layers of roving put on only. The layup must be heat treated (Bud said only to 120F. He mentioned to limit gross to 1370 if you are flying one without this mod. He also I think mentioned to limit max. speed to maneuvering speed. When I asked if I should rig my short wings now or wait till long wing mod his answer was “do it now”!