## wing incidence - glider

| from [Rob Neils] | [Permanent Link] |
| --- | --- |

| Subject: | wing incidence - glider |
| --- | --- |
| Date: | Wed, 5 Jun 2002 11:31:33 |

Since the picture attachment of my last posting to the forum didn't upload

I'll describe in words a way to insure exact dihedral and incidence as you

install the glider wings...or shorter, less adequate wings.

Before you cut out the BL 251 incidence template draw an extension at the

bottom of the template about an inch or so downward to capture both the

leading and trailing edges of the wing.  Surround the wing with the

template board back about 2" from the leading and aft edges.  The template

then partially encircles the wing securing it not only at the top but at

the bottom too. The template board will be able to support the wing.

Note exactly where the leading edge and trailing edge of the template meets

the leading and trailing edges of the actual wing.  This is critical to get

the incidence exact.  The curve of the template over the wing does not have

to be perfect but the spot where the template meets the wing at the

furtherst forward and aft is critical.  In fact, if you have a little

accidental gap between the wing and the template you'll have a little space

into which you can push some glue to hold the incidence board securely to

the wing - eliminating a confounding variable.

Draw two ~1" squares onto the BL 251 incident template just forward and

just aft of where the leading edge and trailing edge of the template meets

the leading and trailing edge of the actual wing.  When you cut out the

template include the 1" squares as part of the template board.  The

template board will have a ~1" X 1"protruberance at the front and back

which will accept the threaded rods which will hold up the wings.

Drill a 1/2" hole perpendicularly down through each of the 1" square

protuberances to accept 1/2" threaded rods.  Make the holes a little too

large so that the rods can slide through the holes without much friction,

but without getty sloppy.  The rods need to be about 4' in length, long

enough to extend from the template to the floor of your workshop.  Don't

skimp and buy 3/8" rods.  Splurge and get the far more expensive 1/2" rods

because 3/8" rods allow too much wobble while you're making measurements

and wobbly rods can let the wing fall...oops!

Spot glue the templates onto each wing so the templates don't

wobble.  Another confounding variable is slain before it has the chance to

bite your fine motorglider.

Insert the rods which now support the wings upwards, forward and aft.

Now you can use a 1/2" nut with a washer to raise and lower the entire wing

to set the 2.4 degrees of dihedral.  You can also raise or lower the

forward or aft edge of each wing to set the 2.0 degress of incidence.  You

can reset a 1/10 degree difference easily by cranking the nut up or down

each rod, pushing the wing up and down.  The weight of the wing will hold

the wing against the set nut and washer on the too elevated side of the

wing.  If it doesn't then use a few weights to press the wing down onto the

washer and nut.

The rods hold everything solidly in place.  Spinning the nuts up and down

allow you to set and securely hold the exact 2.4 degrees of dihedral and

exactly 2.0 degrees of incidence on the glider wings.

I suppose this method would work for you short wing enthusiasts.

Remember - Glider guiders keep it up longer!      ;)

Rob Neils, Ph.D.

Spokane WA   USA

A162

Rob.Neils@xxxxxxxxxxx