Subject:; Re: Europa-List: Re: Europa-List Digest: 2 Msgs - 07/25/11. From:; "Jim Brown" <acrojim7534@YAHOO.COM>. Date:; Tue, July 26, 2011 7:26 am. To:; "europa-list@matronics.com" <europa-list@matronics.com>. Priority:; Normal. Options:; View Full Header ¦ View Printable Version ¦ Download this as a file ¦ Add to Address Book ¦ View Message Details ¦ View as plain text.

Tony I had almost the same problem.. Engine runs fine until the boost started to come on, then missing.

Try this. turn off the turbo with the Isolation switch....Do a run up, if you get full power, and no missing, without the turbo, then leave the turbo out of the circuit and fly home....

The fuel regulator on top of the airbox is bad.  As the engine RPM is going to full power, at about 4400 RPM the missing starts.,the fuel regulator on top of the airbox, is supposed to shut off the return fuel, so that all fuel is avaiable for the turbo. In other words the engine is starving for fuel under boost. The engine will run just fine as a non-turbo.....

Jim Brown

N398JB

From: Tony Renshaw <tonyrenshaw268@gmail.com>
To: europa-list@matronics.com
Sent: Tuesday, July 26, 2011 6:39 AM
Subject: Europa-List: Re: Europa-List Digest: 2 Msgs - 07/25/11

--> Europa-List message posted by: Tony Renshaw <tonyrenshaw268@gmail.com>

I've got a mate, who we all know, Kingsley, trying to fly a 914 a long distance downunder that is giving him angst. Its running rough, real rough. It seems that at boost power it just simply cuts out! Also, in cruise when going to climb power it has begun to run really rough. He is a long way from home trying to limp it home, so any suggestions would be greatly appreciated. To elaborate, he has flown it 2 hours on AVGAS with no obvious problem until going to climb power when it ran so rough they had to pull the power back and land ASAP, luckily with an aerodrome closeby. Now, it runs rough on the ground too….not sure of what power settings but I recall they can't get boost happening. The local Rotax agent says that AVGAS with lead can wreck a set of plugs in no time, so, they have a new set of plugs about to be installed. Fuel will be drained and high octane super unleaded mogas will be used instead. The system will be drained. So, I'm wondering if ASAP those in the know can!
  beam downunder the "good goss" that can help Kingsley get this bird home. 
Reg
Tony Renshaw
Sydney Aussie.