Engine sensor location and wiring information. Europa-List: Re:. From:; "Bud Yerly" <budyerly@msn.com>. Date:; Mon, April 18, 2011 11:18 am. To:; europa-list@matronics.com. Priority:; Normal. Options:; View Full Header ¦ View Printable Version ¦ Download this as a file ¦ Add to Address Book ¦ View Message Details ¦ View as plain text.

Tony,

The Rotax can be kept quite simple as it is using Bing carbs and feeding independent cylinders.

My recommendation for the Rotax is:

Two CHT on Cyl 1 and 3.  Just to monitor the temp extremes of coolest and warmest.

Oil Temp and Pressure using the VDO senders on the Rotax.

Two 2 EGT on the back two cylinders.

This helps determine if one carb is leaning or having problems vs the other when troubleshooting the rough running engine.  It is not necessary and I don't have them on mine as you can hear and feel if the engine is out of balance.

If you use round VDO or Mitchell gauges you do not need to buy new CHT, or Oil and temp/pressure senders. They work fine using the Rotax senders.  To monitor EGT, just purchase the Westach clamp on probe for the Rotax 912 or the bayonette type for the 914 since it has the bosses welded in already...

If you install a Dynon or Grand Rapids or similar EFIS / Engine Monitor system you must buy their probes for CHT and EGT .

Fuel pressure is a must for me as it an ammeter and voltmeter.  If you use a constant speed prop a manifold pressure gauge is essential.

My basic round dial setupfor the 914 is TACH, MAN PRESS, OIL T, OIL P, CHT #3 cyl (one), FUEL PRESS, AMPS&VOLTS (one one gauge by Westach).

Regards,

Bud

----- Original Message -----

From: Tony Renshaw

To: europa-list@matronics.com

Sent: Sunday, April 17, 2011 11:12 PM

Graham and co,

I was thinking in terms if a 4 cylinder Rotax and

Realize your advice pertained to the 6 cylinder

Jabiru and to be able to monitor each cylinder.

(Buenos Aires to Sydney yesterday. Brain is about

Over International dateline, or the Pacific somewhere,

certainly not in my body.)

Reg

Tony Renshaw 

On 18/04/2011, at 5:17 AM, GRAHAM SINGLETON <grahamsingleton@btinternet.com> wrote:

Rick
there's a lot of good experience out there for you to benefit from. (I don't have much) 
I recommend a good engine monitor with 6 EGTs and 6 CHTs, Mixture distribution is tricky and important.
Graham

From: AirEupora <AirEupora@sbcglobal.net>
To: europa-list@matronics.com
Sent: Sunday, 17 April, 2011 18:41:04
Subject: Europa-List: Re: Jabiru vs. Rotax

--> Europa-List message posted by: "AirEupora" <AirEupora@sbcglobal.net>

You are right Kevin.  Trying to compare a Jabiru to a Rotax 912 or 914 is like comparing Apples and Oranges.  Both taste good.  

The Jabiru will use a little over one gallon more per hour so it will cost me $5.00 more an hour to operate.  If I get to fly 100 hour this coming year it will cost me $500.00 dollars this year over your cost for fuel.  I can still divide that into the initial cost firewall forward.

I looked on the web pages today and Rotax 912ULS is 21K plus the cost of firewall forward vs. 18K for everything on the Jabiru.  

Rick Stockton




Read this topic online here:

http://forums.matronics.com/viewtopic.php?p=337229#337========================http://www.matronics.com/Navigator?Eurp; -->

==========

courier new,courier">http://www.matronics.com/Navigator?Europa-List

==========

ums.matronics.com">http://forums.matronics.com

==========

http://www.matronics.com/contribution">http://www.matronics.com/contribution

==========

href="http://www.matronics.com/Navigator?Europa-List">http://www.matronhref="http://forums.matronics.com">http://forums.matronics.com

href="http://www.matronics.com/contribution">http://www.matronics.com/c